| SoftAP Mode - features | 8188cu/8192cu |
| --- | --- |
| Hosted Daemon | hostapd v0.6.9 + Realtek proprietary driver interface for 871x-based |
| Support MAX. number of stations | 8 Clients |
| No Security | yes |
| Security Mode - WPA2-AES - PSK | yes |
| Wireless Mode - B/G/N | yes |
| a. fragmentation b. defragmentation | a. No b. Yes |
| 802.11N HT – Band Width 20/40 MHZ | yes |
| 802.11N HT - AMPDU TX-AMPDU RX-AMPDU | Tx: yes Rx: yes |
| Intra-BSS distribution | yes |
| legacy power saving | yes |
| WPS v1.1 - internal registrar | yes, PIN/PBC Methods |